Sea State along-track time series of key parameters
Version 4, 4/23/2017

O. Persson

MATLAB file: SeaState_2015_met_sfc_flx_V4_10min

netCDF file: SeaState_2015_met_sfc_flx_V4_10min.cdf

Parameters in file SeaState_2015_met_sfc_flx_V4_10min

There is both a matlab version and a NetCDF version of this file.  They contain the same variables.

File name shows that values are 10-minute averages (or interpolations for wave parameters) for Days 275 (Oct 2) through 309 (Nov 5) 2015 along track of R/V Sikuliaq.

% col varName description

% 1 jd_ref decimal day-of-year at start of averaging interval

% 2 lat_ref latitude

% 3 lon_ref longitude, +/- 180 deg

% 4 sog_ref gps speed over ground, m/s

% 5 std_sog_ref std deviation in sog, m/s

% 6 cog_ref gps course over ground, deg

% 7 hed_ref gps heading, deg

% 8 std_hed_ref std deviation in gyro heading, deg

% 9 wspd_ref true windspeed, composite, m/s

% 10 wdir_ref true wind direction, composite, m/s

% 11 rwspd_ref relative wind speed, composite, m/s

% 12 std_rwspd_ref std deviation rel wind speed, composite, m/s

% 13 rwdir_ref relative wind dir, composite, +/- 180 deg from bow

% 14 std_rwdir_ref std deviation rel wind direction, composite, deg

% 15 ta_ref air temperature, C

% 16 rh_ref relative humidity, %

% 17 qa_ref specific humidity, g/kg

% 18 ts_skn_ref composite skin temperature from ship-based IR sources, deg C

% 19 ts_snk_ref sea-snake temperature when deployed (either at 10 cm depth or on top of ice/snow), deg C

% 20 to_frz_ref freezing point of sea water from salinity from ship intake (6.5 m depth), deg C

% 21 sal_ref salinity at ship intake (6.5 m depth), PSU

% 22 qs_ref surface saturation specific humidity, g/kg

% 23 p_mb_ref atmospheric pressure at height zp, mb

% 24 slp_ref sea-level pressure by height-correcting p_mb_ref, mb

% 25 swd_med_ref downwelling SW radiation, manually edited, W/m2

% 26 swd_bst_ref downwelling SW radiation, linear interpolation across gaps, W/m2

% 27 lwd_med_ref downwelling LW radiation, manually edited, W/m2

% 28 lwd_bst_ref downwelling LW radiation, gaps estimated from ceilometer & soundings, W/m2

% 29 mlh_ref atmospheric mixed-layer height at times of soundings, m

% 30 cld_bas_ref median cloud base from ceilometer, m

% 31 cld_frc_ref cloud fraction(%); percent of time within 10-min period that ceilometer detected a cloud

% 32 pcp_ref  precipitation rate (set to 0.1 mm/h when logs indicated precip; otherwise 0)

% 33 zt_ref air temperature measurement height, m

% 34 zq_ref air humidity measurement height, m

% 35 zp_ref air pressure measurement height, m

% 36 zu_ref wind speed measurement height, composite

% 37 hsc_ref sensible heat flux -covariance (W/m2)

% 38 hsid_ref sensible heat flux - inertial dissipation (W/m2)

% 39 hs_blkr sensible heat flux - bulk calculated using COARE and SHEBA flux(W/m2)

% schemes depending on mosaic method & mean ice concentration. Neither scheme was developed for mixed waves and ice.

% 40 hlc_ref latent heat flux - covariance (W/m2)

% 41 hlid_ref latent heat flux - inertial dissipation (W/m2)

% 42 hl_blkr latent heat flux -bulk as for sensible heat flux (W/m2)

% 43 tauc_ref stress - covariance (N/m2)

% 44 tauid_ref stress - inertial dissipation (N/m2)

% 45 tau_blkr bulk stress (N/m2), as for sensible heat flux

% 46 swu upwelling shortwave radiation estimated from best downwelling SW radiation and estimated surface albedo & mosaic method with ice concentration; W/m2

% 47 lwu upwelling longwave radiation estimated from composite skin temperature, estimated surface emissivity, and Stefan-Boltzmann relation& mosaic method with ice concentration; W/m2

% 48 alb_ref surface albedo estimated from the observed surface conditions(ice conc, snow depth, skin temperature, water freezing point) and subjective estimates of alb=0.08 (open water), 0.35 (thin ice), 0.65

% (thicker ice), 0.85 (snow covered ice)

% 49 emiss_ref surface emissivity estimated from surface type/conditions: emiss=0.99 (open water), 0.985 (ice covered water)

% 50 fatm net atmospheric energy flux(=swd_bst_ref-swu+lwd_bst_ref-lwu-hs_blk_ref-hl_blk_ref); W/m2

% 51 Ai best estimate of ice concentration;%

% 52 ice_concvo_ref total ice concentration from visual observations,0-10

% 53 ice_typ1vo_ref primary ice type from visual observations

% 54 ice_z_ref ice thickness estimates from visual observations, cm

% 55 snow_z_ref snow depth estimates from visual observations, cm

% 56 floe_size_ref ASSIST floe size code for ice type with greatest concentration(100-pancakes;200-new sheet ice;300-Brash/broken ice;

%    400-cake ice,<20m;500-smallfloes,20-100m;600-medium floes, 100-500 m; 700-large floes, 500-2000 m;800-vast floes, >2000m; 900-bergy floes

% 57 ziceSEBmd_ref - 10-min median of 1-s ice thickness from thermodynamic technique (m), mean of two KT-15 radiometers

% 58 zicesig_ref - 10-min standard deviation of 1-s ice thickness values from TD technique (m)

% 59 iceconcSEB_ref - 10-min ice concentration from thermodynamic technique(%)

% 60 zice_dpnt_ref - ice thickness from dipnet (P. Wadhams) (m)

% 61 iconcsh_amsr_ref - ice concentration from daily AMSR2 (3.125 km res)(%)

% 62 iedis_ref - distance to nearest ice edge (km)

% 63 ieaz_ref - azimuth to nearest ice edge (deg)

% 64 ieor_ref - orientation of nearest ice edge (deg)

% 65 Hsig_Rgl_ref - significant wave height from 1-D Riegl (m)

% 66 Tpi_Rgl_ref - max wave period from 1-D Riegl(s), Doppler corrected ala (Collins et al 2017)

% 67 radar_wave_tp_ref - marine radar peak wave period (s) (B. Lund)

% 68 radar_wave_dp_ref - marine radar peak wave direction (deg clockwise from N), coming from (B. Lund)

% 69 radar_sfccurr_spd_ref - marine radar surface current speed (m/s) (B. Lund)

% 70 radar_sfccurr_dir_ref - marine radar surface current direction (deg clockwise from N), coming from (B. Lund)

% 71 radar_icedrift_spd_ref - marine radar ice drift speed (m/s) (B. Lund)

% 72 radar_icedrift_dir_ref - marine radar ice drift direction (deg clockwise from N), coming from (B. Lund)

% 73 omld_ref - ocean mixed-layer depth from uCTDs (m)

% 74 omlheat_ref - heat content of ocean mixed-layer (J/m2)

% 75 omlxcsheat_ref - excess heat content (above freezing point) of ocean mixed-layer (J/m2)

% 76 omlp15heat_ref - heat content of ocean layer between OML and OML+15 m (J/m2)

% 77 ctd6m_T_ref - CTD temperature at 6 m (deg C)

% 78 ctd6m_S_ref - CTD salinity at 6 m (psu)

% 79 ctd6m_frzT_ref - freezing point at 6 m depth (deg C)